###### Van’s Aircraft RV Super 8

###### Pilot’s Operating Handbook

###### N687MS

Note: This POH is for this plane only. This plane is highly modified from the designers original specifications. Its of higher power and weight. Both these items should be carefully considered when flying this experimental. Speed, G’s, aerobatics, and stick force should all be monitored carefully. This airplane significantly reduces the designers built in safety factor. DO NOT take this lightly. You are flying this plane at your own risk. The builder can not emphasize enough the reduced safety margins of this aircraft as a result of the big engine. Be Safe and conservative when flying this wonderful machine.

Van’s Aircraft RV Super 8	1

Pilot’s Operating Handbook	1

N687MS	1

1

PERFORMANCE – SPECIFICATIONS	5

SPEED	7

VNO	7

85 Kts	7

AIRSPEED INDICATOR MARKINGS (std. RV-8)	7

AEROBATIC INFORMATION	7

PREFLIGHT INSPECTION	9

BEFORE STARTING ENGINE	11

STARTING ENGINE (cold)	11

STARTING ENGINE (Warm)	11

RUN-UP	11

TAKEOFF	13

NORMAL TAKEOFF	13

SHORT FIELD TAKEOFF	13

ENROUTE CLIMB	13

CRUISE	14

LANDING	14

AFTER LANDING 	14

POSTFLIGHT	15

ENGINE SHUTDOWN	15

SECURING AIRCRAFT	15

Performance	16

Cruise Performance at 10,000’: ROP 2450rpm	16

Cruise Performance at 8,000’: LOP 2450rpm	16

Cruise Performance at 8,000’: LOP 2350rpm	17

No Wind Range at 8,000’: Lean of Peak Operations	17

WEIGHT AND BALANCE DATA	18

EMERGENCY PROCEDURES	19

AIRSPEEDS FOR EMERGENCY OPERATIONS	19

ELECTRICAL / ALTERNATOR FAILURE	19

Items available on Essential Buss Only	19

Items available on Instrument Buss	19

Items available on Main Buss (Master sw on)	19

ENGINE FAILURES	21

FIRES	21

FAR 91.125 - ATC light signals.	23

Compass Headings, VFR under 18,000ft	23

# PERFORMANCE – SPECIFICATIONS

SPAN: ………………………………………………………………..	24’ 0”

LENGTH: …………………………………………………………….	21’

HEIGHT:  …………………………………………………………….	5’ 7”

Wing Area……………………………………………………………..	116ft2

SPEED:

Maximum at Sea Level ……………………………………..	206 iKTAS

Cruise, 75% Power at 10,000 Ft ……………………………..	197 iKTAS

Range

1/2 hour (6 gal.) reserve: LOP Operations

WOT 2450RPM………………………………………		748 nm

WOT 2350RPM………………………………………		785 nm

MP 17.4, ITAS 167, 2350rpm……………………….		860 nm

RATE OF CLIMB AT SEA LEVEL Solo………………………….….	2,800fpm

SERVICE CEILING Demonstrated…………………………..…….	29,000' FT

TAKEOFF PERFORMANCE: ……………. ……………….………	500 Ft

LANDING PERFORMANCE: ………………..…………….………	800 Ft

STALL SPEED (CAS):

Flaps Up, Power Off ………………………………………	55 Knots

Flaps Down, Power Off ……………………………………	50 Knots

MAXIMUM WEIGHT (Normal Category): ………………………….	2270 Lbs

EMPTY WEIGHT ………………………………………….		1326 Lbs

MAXIMUM USEFUL LOAD: ……………………………………..	944 Lbs

BAGGAGE ALLOWANCE Aft/Fwd…………………………………	100/60 Lbs

WING LOADING (Pounds/ Sq. Ft) …………………………………	19.5

POWER LOADING (Pounds/ HP) ………………………………….	8.4

FUEL:

Capacity/Usable gal.………………………………		60/59

Type …………………………………………......      	     100LL/87oct

OIL CAPACITY in Qts………………………………………		12, 10 normal

ENGINE: Lycoming 6cyl 260hp, Fuel Inj., Elect. Ign……… 	IO540-C4B5

Propeller: Hartzell 76” Aztec Blades chopped to 76”...	HC-C2YR-1B

AIRSPEED LIMITATIONS

|  | SPEED | KIAS | REMARKS |
| --- | --- | --- | --- |
| VNE | Never Exceed Speed | 200 Kts | Do not exceed this speed in any operations. |
| #### VNO | Maximum Structural Cruising Speed | 156 Kts | Exceed this speed only in smooth air. |
| VA | Maneuvering Speed | 115 Kts | Do not make full control movements above this speed. |
| VFE | Maximum Flap Extended Speed | 96 –20 deg 87 - Full | Do not exceed this speed with flaps down |
| Vy | Best Rate of Climb | #### 85 Kts |  |
| Vx | Best Angle of Climb | 75 Kts |  |
| Vs | Stall Speed Clean | 55 Kts |  |
| Vso | Stall Speed Landing Configuration | 50 Kts |  |

# AIRSPEED INDICATOR MARKINGS (std. RV-8)

| MARKING | KIAS VALUE OR RANGE | SIGNIFICANCE |
| --- | --- | --- |
| White Arc | 43 – 87 Kts | Full Flap Operating Range. |
| Green Arc | 47 – 156 Kts | Normal Operating Range. |
| Yellow Arc | 170 – 200 Kts | smooth air. |
| Red Line | 200 Kts | Maximum speed |

# AEROBATIC INFORMATION

Weight Limitation – 1600 Pounds 6G’s, 1800 5G’s

Recommended Entry Speeds:

Loops, Cuban Eights 		140 – 165 Kts

Immelman Turns 		130 – 165 Kts

Aileron Rolls, Barrel Rolls 	105 – 165 Kts

Snap Rolls			70  –   95 Kts

Vertical Rolls 		155 –165 Kts

Split-S 			85  -  95 Kts

# PREFLIGHT INSPECTION

1.	CABIN

- Documentation -- Available In Airplane

- Aeronautical Charts – CURRENT AND APPROPRIATE TO FLIGHT

- Seat Belt Securing Control Stick --  RELEASE

- Ignition Switch  -- OFF

- Avionics -- OFF

- Master Switch -- ON

- Engine Monitor – ON

- Fuel Quantity -- CHECK QUANTITY
			      -- VERIFY ENGINE MONITOR PRESET

- Flaps – DOWN

- Master Switch -- OFF

2. EMPENNAGE

- Tail Tie-Down -- DISCONNECT

- Control Surfaces -- CHECK freedom of movement and security

3. RIGHT WING

- Aileron -- CHECK freedom of movement and security

- Flap -- CHECK security

- Wing Tie-Down -- DISCONNECT

- Main Wheel Tire -- CHECK for proper inflation

- Chock -- REMOVE

- Right Wing Tank – SUMP

- Fuel Quantity -- CHECK VISUALLY

- Fuel Filler Cap – SECURE

5. NOSE

- Engine Oil Level -- CHECK, do not operate with less than 4 quarts

- Propeller and Spinner -- CHECK for nicks and security

- Cowl Hinge Pins – CHECK for security

- Air Inlet -- CHECK for restrictions

- Chock -- REMOVE

- Fuel Tank Vents -- CHECK for blockage

6. LEFT WING

- Wing Tie-Down -- DISCONNECT

- Main Wheel Tire -- CHECK for proper inflation

- Chock -- REMOVE

- Left Wing Tank -- SUMP

- Fuel Quantity -- CHECK VISUALLY

- Fuel Filler Cap -- SECURE

- Pitot Tube Cover -- REMOVE and check for blockage

- Landing Light -- CHECK condition

- Aileron -- CHECK freedom of movement and security

- Flap -- CHECK security

BEFORE STARTING ENGINE

- Preflight Inspection -- COMPLETE

- Seat Belts and Shoulder Harnesses -- ADJUST and LOCK

- Fuel Selector Valve -- DESIRED TANK

- Avionics and Electrical – ALL OFF

- Brakes -- SET

- Canopy adjust

STARTING ENGINE (cold)

- Mixture -- LEAN

- Throttle to 1/8 (crack)

- Prop -- HIGH RPM

- Master Switch – ON

- Alt Field On

- Flaps – UP

- Brakes Set

- Fuel Boost Pump -- ON

- Prime – Mix to rich (5-7 seconds when cold)

- Fuel Boost Pump – OFF,  Propeller Area – CLEAR

- Crack Throttle

- L&R Ignitions ON

- Ignition Switch – START 900rpm

- Instruments, Essential & APRS switches – ON

- Oil Pressure -- CHECK   >50  psi at idle

- Strobe – ON

- Noise Canceling On

- EFIS Accept, GPS Ent, Ent

STARTING ENGINE (Warm)

- Mixture – Lean ,Throttle to 1/8

- Prop -- HIGH RPM

- Master Switch – ON

- Alt. Field

- Flaps – UP,   Brakes Set

- Propeller Area -- CLEAR

- Ignition Switch – START

- Count 4 blades,  mixture rich

- Instruments, Essential & APRS switches– ON

- Oil Pressure -- CHECK   >50  psi at idle

- Strobe – ON,   GPS Ent, Ent

- Noise Canceling On

- EFIS Accept, GPS Ent, Ent

RUN-UP

Brakes -- SET

- Canopy  -------- Main Latch – SECURE

- Flight Controls -- FREE and CORRECT

- Flaps -  UP

- Flight Instruments – SET
      Altimeter(both) – CORRECT PRESSURE

Engine Instruments on the numbers

- Fuel Selector Valve -- DESIRED TANK

- Mixture -- RICH (below 3000’)

- Elevator and Aileron Trim -- (elev just off the bottom)

- Throttle -- 1700 RPM

- Magneto/EI --  CHECK (125 max drop)

- Engine Instruments – CHECK ALL

- Throttle -- IDLE

- Radios – SET

- Trim spd sw high rate (toggle Up)

- Fuel Boost Pump -- ON

- Transponder – (GPS activated when moving)

- Passenger – READY and willing

# TAKEOFF

Oddities:

- Left fuel Gage may go to zero on occasion

- #2egt may spike up on occasion

## NORMAL TAKEOFF

- Wing Flaps – UP

- Prop – HIGH RPM

- Throttle --  FULL OPEN

- Climb Speed --  110 KIAS

## SHORT FIELD TAKEOFF

- Wing Flaps – 10 Deg

- Prop – HIGH RPM

- Brakes – APPLY

- Elev full back

- Throttle – 2300 RPM

- Mixture – RICH (above 3000’ lean to obtain max RPM)

- Brakes – RELEASE

- Full Throttle, Release back pressure

- Climb Speed – 80 KIAS

## ENROUTE CLIMB

- Airspeed – 110 - 140 KIAS

- Throttle – Full

- Prop – 2500 RPM

- Boost Pump – OFF at 1000 feet AGL

- Fuel Pressure – CHECK

- Mixture – LEAN above 3000’

## CRUISE

- Throttle – FULL

- Prop – 2300 - 2450 RPM

- Adjust trim

- Mixture – LEAN to 50deg  ROP, or 40 LOP

- Adjust trim

## LANDING

- Mixture Rich, Prop High RPM

- Trim spd sw high rate (toggle Up)

- Landing lights on

- Pattern spd 100 KIAS

- Fuel Boost Pump On

- Adj. elev trim constantly as needed

- ¼ flaps abeam the numbers

- ½ flaps on base, Full on FInal

- Base to final 90 KIAS

- Over the numbers at 75 KIAS

## AFTER LANDING

- Wing Flaps – Leave Down

- Boost Pump – OFF

- Landing Lights off

- Elevator Trim -- (elev just off the bottom)

# POSTFLIGHT

## ENGINE SHUTDOWN

- Flaps – DOWN

- Prop – FULL FORWARD

- Throttle – IDLE

- Mixture Idle cut off

- Immediately shut off ignition while blades turning

- Instruments and Avionics SW.—OFF

- All electrical sw  – OFF

- Master – OFF

## SECURING AIRCRAFT

- Wheel Chocks

- Wing & Tail Tie-Down if necessary

- Lock flight controls with Pilot Seat Belt

- Master and Electrical Switches – OFF

- Rudder locked with tail wheel

# Performance

## Cruise Performance at 10,000’: ROP 2450rpm

| FF | 13.8 | 13.4 | 12.8 | 12.1 | 11.2 | 10.7 | 9.6 | 8.7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ITAS | 197 | 192 | 187 | 180 | 176 | 169 | 159 | 143 |
| IAS | 167 | 164 | 160 | 154 | 150 | 144 | 136 | 122 |
| MP | 20.5 | 19.4 | 18.5 | 17.5 | 16.5 | 15.3 | 14 | 12.2 |
| RPM | 2450 | 2450 | 2450 | 2450 | 2450 | 2450 | 2450 | 2450 |
| KTS/Gal | 14.28 | 14.33 | 14.61 | 14.88 | 15.71 | 15.79 | 16.56 | 16.44 |

## Cruise Performance at 8,000’: LOP 2450rpm

| FF | 12.5 | 12.1 | 11 | 10.2 | 9.4 | 8.7 | 7.6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ITAS | 187 | 185 | 181 | 173 | 164 | 155 | 141 |
| IAS | 164 | 162 | 158 | 150 | 145 | 136 | 124 |
| MP | 21.6 | 20.4 | 19.3 | 17.9 | 16.3 | 14.9 | 13.1 |
| RPM | 2450 | 2450 | 2450 | 2450 | 2450 | 2450 | 2450 |
| KTS/Gal | 14.96 | 15.29 | 16.45 | 16.96 | 17.45 | 17.82 | 18.55 |

## Cruise Performance at 8,000’: LOP 2350rpm

| FF | 11.9 | 11.4 | 10.3 | 9.7 | 8.5 | 7.6 |
| --- | --- | --- | --- | --- | --- | --- |
| ITAS | 187 | 180 | 176 | 167 | 158 | 145 |
| IAS | 164 | 157 | 154 | 147 | 138 | 127 |
| MP | 21.6 | 20.1 | 19 | 17.4 | 15.3 | 13.6 |
| RPM | 2350 | 2350 | 2350 | 2350 | 2350 | 2350 |
| KTS/Gal | 15.71 | 15.79 | 17.09 | 17.22 | 18.59 | 19.08 |

## No Wind Range at 8,000’: Lean of Peak Operations

- All range calculations include 3 gal. for engine start, taxi, takeoff and climb. Engine is leaned for best economy LOP.

1/2 hour (6 gal.) reserve:

WOT 2450RPM………………………………..	748 nm

WOT 2350RPM………………………………..	785 nm

MP 17.4, ITAS 167, 2350rpm…………………	860 nm

No Reserve:

WOT 2450RPM………………………………..	837 nm

WOT 2350RPM………………………………..	880 nm

MP 17.4, ITAS 167, 2350rpm…………………	964 nm

Oxygen Cylinder Chart Usage using flow meter and O2 saving canula

Typical usage, @15k' 1.5L/min/person, @25k' 2.5L/min/person

# WEIGHT AND BALANCE DATA

| WEIGHT AND BALANCE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ---Weighing Procedures--- |  |  |  |  |  |  |  |  |
| Date: |  |  | 8.07.2005 |  |  |  |  |  |
| Aircraft Type: |  |  | RV8 N687MS |  |  |  |  |  |
| Your Name: |  |  | Michael Stewart |  |  |  |  |  |
| DATUM: |  |  | 70" forward wing leading edge |  |  |  |  |  |
| CG Limits - 78.7 to 86.82 |  |  |  |  |  |  |  |  |
| 78.7 79.7 80.7 81.7 82.7 83.7 84.7 85.7 86.82 |  |  |  |  |  |  |  |  |
|  | Scale |  |  |  |  | C of G |  |  |
| Measured Weights | Reading | Weight |  | STA | Moment | Location |  |  |
| Left wheel | 628.00 | 628.00 |  | 68.76 | 43181.28 |  |  |  |
| Right wheel | 633.00 | 633.00 |  | 68.76 | 43525.08 |  |  |  |
| Nose or tail wheel 43 | 65.00 | 65.00 |  | 251.06 | 16318.90 |  |  |  |
| BEW (Basic Empty Weight) |  | 1326.00 |  |  | 103025.26 | 77.70 |  |  |
| Solo Configuration | Gallons |  |  |  |  |  |  |  |
| --Right Fuel Tank (No. of gallons & STA): | 30.00 | 180.00 |  | 70.00 | 12600.00 |  |  |  |
| --Left Fuel Tank (No. of gallons & STA): | 30.00 | 180.00 |  | 70.00 | 12600.00 |  |  |  |
| --Pilot: | -- | 200.00 |  | 91.78 | 18356.00 |  |  |  |
| --CoPilot: | -- | 0.00 |  | 119.12 | 0.00 |  |  |  |
| --Smoke | -- |  |  |  |  |  |  |  |
| --Aft Baggage Shelf | -- | 21.00 |  | 152.91 | 3211.11 |  |  |  |
| --Aft Baggage Floor | -- | 0.00 |  | 138.00 | 0.00 |  |  |  |
| --Fwd Baggage |  | 0.00 |  | 56.75 | 0.00 |  |  |  |
| Empty Weight |  | 1326.00 |  |  | 103025.26 |  |  |  |
| Totals Fuel Full |  | 1907.00 |  |  | 149792.37 | 78.55 |  |  |
| Totals Zero Fuel |  | 1547.00 |  |  | 124592.37 | 80.54 |  |  |
| Pax and Baggage Configuration | Gallons |  |  |  |  |  |  |  |
| --Right Fuel Tank (No. of gallons & STA): | 30.00 | 180.00 |  | 70.00 | 12600.00 |  |  |  |
| --Left Fuel Tank (No. of gallons & STA): | 30.00 | 180.00 |  | 70.00 | 12600.00 |  |  |  |
| --Pilot: | -- | 200.00 |  | 91.78 | 18356.00 |  |  |  |
| --CoPilot: | -- | 175.00 |  | 119.12 | 20846.00 |  |  |  |
| --Smoke | 7.00 | 49.00 |  | 97.00 | 4753.00 |  |  |  |
| --Aft Baggage Shelf | -- | 20.00 |  | 152.91 | 3058.20 |  |  |  |
| --Aft Baggage Floor | -- | 80.00 |  | 138.00 | 11040.00 |  |  |  |
| --Fwd Baggage |  | 60.00 |  | 56.75 | 3405.00 |  |  |  |
| Empty Weight |  | 1326.00 |  |  | 103025.26 |  |  |  |
| Totals Fuel Full | Gross Wt | 2270.00 |  |  | 189683.46 | 83.56 |  |  |
| Totals Zero Fuel |  | 1861.00 |  |  | 159730.46 | 85.83 |  |  |

# EMERGENCY PROCEDURES

## AIRSPEEDS FOR EMERGENCY OPERATIONS

Engine Failure After Takeoff:

Wing Flaps Up …………………………	80 Kts

Wing Flaps Down ……………………..	70 Kts

Maneuvering Speed (Va) ……………………….	115 Kts

Maximum Glide ………………………………… 	80 Kts

## ELECTRICAL / ALTERNATOR FAILURE

Total Panel Power Failure

- Engage emergency power toggle Switch Up (lower Rt. Gear Tower)

- This engages Essential Power to run Essentials only (see below)

- If this fails to put power to the panel

  - Engage emergency power to G3 EFIS with switch next to the G3 EFIS. This provides 1.5 hours of power to that unit only

Alternator Failure

- Turn off instruments switch on rt panel

- This keeps essential items running

## Items available on Essential Buss Only

Com 2 SL40, EFIS BMA, GPS1 430, Instrument Lighting, Transponder. Important Items Lost (Eng. Monitor, trim, flaps, fuel pump, pitot heat)

## Items available on Instrument Buss

Com 1 Nav 1 430, EFIS GRT 1&2, Engine Monitor

## Items available on Main Buss (Master sw on)

Landing Strobe Nav Lts, Flaps, Auto Pilot, Trim, Cabin lts, Alt Field, Smoke, XM wx, Fuel Pump, Pitot Heat, Pax GPS

## ENGINE FAILURES

ENGINE FAILURE DURING TAKEOFF RUN

- Throttle –IDLE

- Brakes – APPLY

- Wing Flaps – RETRACT

- Mixture – IDLE CUT-OFF

- Ignition Switch – OFF

- Master Switch – OFF

ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF

- Airspeed – 70 KIAS

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Ignition Switch – OFF

- Wing Flaps – AS REQUIRED

- Master Switch – OFF

ENGINE FAILURE DURING FLIGHT

- Airspeed –80 Knots

- Boost Pump – ON

- Fuel Selector – SWITCH TANKS

- Mixture – RICH

- Ignition Switch – BOTH, LEFT, RIGHT

- Transponder – 7700

## FIRES

DURING START ON GROUND

- Cranking – CONTINUE, to get a start which would suck the flames and accumulated fuel through the engine.

If engine starts:

- Power – 1700 RPM for a few minutes

- Engine – SHUTDOWN and inspect for damage

If engine fails to start:

- Throttle – FULL

- Mixture – IDLE CUT-OFF

- Cranking – CONTINUE

- Fire Extinguisher – OBTAIN

- Engine – SECURE

ENGINE FIRE IN FLIGHT

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Master Switch – OFF

- Cabin Heat and Air – OFF

- Slow to 80kts and continue to build airspeed until fire is out

ELECTRICAL FIRE IN FLIGHT

- Master Switch – OFF

- Avionics – OFF

- All Other Switches (except ignition) – OFF

- Vents/ Cabin Air/ Heat – CLOSED

- Fire Extinguisher – ACTIVATE (if available)

- Engage G3 emergency power switch

CABIN FIRE

- Master Switch – OFF

- Vents/ Cabin Heat – CLOSED

- Fire Extinguisher – ACTIVATE (if available)

WING FIRE

- Nav & Strobe Lights – OFF

- Landing Light – OFF

# FAR 91.125 - ATC light signals.

ATC light signals have the meaning shown in the following table:

| Color and type of signal | Meaning with respect to aircraft on the surface | Meaning with respect to aircraft in flight |
| --- | --- | --- |
| Steady green | Cleared for takeoff | Cleared to land |
| Flashing green | Cleared to taxi | Return for landing |
| Steady red | Stop | Give way to other aircraft and continue circling. |
| Flashing red | Taxi clear of runway in use | Airport unsafe—do not use |
| Flashing white | Return to starting point on airport | N/A |
| Alternating red and green | Exercise extreme caution | Exercise extreme caution |

# Compass Headings, VFR under 18,000ft

| Course | Altitude |
| --- | --- |
| 0 – 179 degrees | Odd thousand +500 |
| 180 – 360 degrees | Even thousand + 500 |

Sample Flight Plan Layout. (used for filing in flight)